ANALYSIS NOTES

Overview

The PCAP analysis focused on reconstructing attacker activity within a simulated water treatment facility environment. Network traffic was analysed to identify indicators of reconnaissance, lateral movement, command-and-control communication, and attempted interaction with industrial control systems (ICS).

The investigation simulated a hybrid cyberattack scenario targeting critical national infrastructure.

Key Findings

Reconnaissance Activity

Analysis identified repeated connection attempts to multiple internal hosts and services. The traffic pattern was consistent with network scanning and internal reconnaissance performed after initial access.

Observed activity included:

- TCP SYN scanning

- Enumeration of SMB, RDP, SSH, and HTTP services

- Discovery of ICS-related communication ports

This behaviour suggests the attacker was mapping the environment to identify accessible systems and pivot points.

Lateral Movement

SMB and RDP traffic indicated possible lateral movement across internal systems.

Observed indicators included:

- SMB session setup requests

- File share access attempts

- Internal remote desktop communication

These techniques are commonly associated with post-compromise movement following credential theft or privilege escalation.

ICS / OT Communication

Traffic on TCP port 502 indicated Modbus communication between internal hosts and industrial control systems.

Analysis identified:

- Modbus requests

- PLC communication attempts

- Potential controller interaction

The presence of Modbus traffic suggests attempted access to operational technology (OT) systems within the environment.

Potential Sabotage Attempts

Review of Modbus function codes indicated possible write operations directed toward PLC devices.

These actions may represent attempts to:

- Modify process parameters

- Change controller behaviour

- Interfere with industrial operations

Although no persistent operational changes were confirmed, the observed traffic demonstrated attempted interaction with control systems.

Command and Control (C2)

External communication analysis identified repeated outbound connections to external IP addresses.

Observed behaviour included:

- Persistent low-volume TCP traffic

- Repeated external communication intervals

- Beaconing-like behaviour

This traffic pattern is commonly associated with command-and-control activity used by remote access tools or malware.

Security Observations

The investigation highlighted several security weaknesses commonly associated with critical infrastructure environments:

- Weak IT/OT network segmentation

- Unauthenticated ICS protocols

- Excessive trust between internal systems

- Limited monitoring of industrial communication

- Insufficient detection of lateral movement activity

These weaknesses increase the risk of attackers moving from corporate environments into operational technology systems.

Defensive Recommendations

Recommended security improvements include:

- Strong segmentation between IT and OT networks

- Multi-factor authentication for privileged accounts

- Continuous monitoring of ICS protocols

- Detection rules for Modbus write operations

- Restriction of unnecessary SMB and RDP access

- Centralised logging and threat detection

Conclusion

The PCAP investigation demonstrated how attackers can combine credential compromise, internal reconnaissance, lateral movement, and industrial protocol abuse to target critical infrastructure environments.

The analysis reinforced the importance of proactive monitoring, segmentation, secure access controls, and ICS-aware threat detection strategies.

Disclaimer

This analysis was conducted in a controlled academic environment for cybersecurity research and educational purposes only.